# Customer name: JSC «Riga Electric Machine Building Works»

(AS «Rīgas elektromašīnbūves rūpnīca»)

Legal address: Ganibu Dambis 53, Riga, Latvia, LV-1005

Contact person: Vladimir Malgin, Chief Technologist

Telephone for contacts: + 371 27000891

E-mail: vladimir.malgin@rer.lv, dt@rer.lv

# Technical requirements for the supply of anodized aluminium plaques production line

# Pamatprasības

| # № | # Parameter | # Requirement |
| --- | --- | --- |
| # Common parameters |  |  |
| # 1. | Application area | Full production (printing and cutting) of 0.5-1 mm thick anodized aluminium plaques. |
| # 2. | # Completeness of delivery | Production line delivery must include a printer for anodized aluminium printing with a vacuum table, a thermal bath, milling equipment with a vacuum table, as well as software necessary for working with the equipment, a starter set of consumables (ink, aluminium sheets etc). All auxiliary materials necessary for installation and start-up of equipment (cables, etc.), commissioning and personnel training should be included in the basic cost. |
| # 3. | # Conformity to EU standards | # The equipment of the production line must have CE marking and conform to EU safety standards. |
| # 4. | # Documentation | The documentation should include equipment operation, maintenance and repair manuals. The language of the technical documentation is Russian. |
| # 5. | # Warranty | # Not less than 12 months from launching day. |
| # Printer for anodized aluminium printing |  |  |
| # 1. | # Printing area dimensions | Not less than 300x500 mm. |
| # 2. | # Type of printing | Piezoelectric inkjet. |
| # 3. | # Type of ink | # Solvent CMYK, 8 cartridges, continuous feed system. |
| 4. | # Additional requirements | # A vacuum table that fits to printing area dimensions. Printing area heating system. Height adjustment system. |
| Thermal bath |  |  |
| 1. | # Working area dimensions | Should fit to printing area and aluminium sheets to be used dimensions. |
| Milling equipment |  |  |
| 1. | # Milling area dimensions | Not less than 300x500 mm. |
| 2. | # Additional requirements | A vacuum table that fits to milling area dimensions. |