# Customer name: JSC «Riga Electric Machine Building Works»

(AS «Rīgas elektromašīnbūves rūpnīca»)

Legal adress: Ganību dambis 53, Rīga, Latvija, LV-1005

Contact personVladimir Malgin, Manufacturing manager

Contact phone: + 371 27000891

e-mail: vladimir.malgin@rer.lv; dt@rer.lv

# Requirements specification for painting chamber with dry filters

# Technical specifications

| - № | - Parameter | - Requirements |
| --- | --- | --- |
| - 1. | Chamber (booth) | - For painting with liquid paints and varnishes. - Pass through type. - With dry filters. - Must have an electrical control panel. |
| - 2 | Paint booth requirements | - Must be manufactured in accordance with ATEX (Zone 2) requirements. - The surface of the chamber is resistant to corrosion. - Must be made of corrosion-resistant material |
| - 3 | Requirements for the control panel of the paint booth | - Must be manufactured in accordance with ATEX (Zone 2) requirements. - There should be an alarm (visual, audible) to control the dust content of the filter. - Automatic shutdown of the spray gun in case of emergency. |
| - 4 | Ventilation requirements | - The exhaust ventilation must have a system of air filtration from paint dust and ensure maximum air purification. - The exhaust ventilation must be equipped with an ATEX centrifugal explosion-proof fan with low noise load. |
| - 5 | Requirements for the light source in the work area | - Illumination not less than 750 lux; - Explosion-proof lamps in accordance with ATEX requirements. |
| - 6 | Chamber noise level | No more than 75 dB |
| - 7 | Overall dimensions of the painted product | Width: 1200mm Length: 1200mm Height: 950 mm |
| - 8 | Weight of the product to be painted, no more than | 1650 kg |
| - 9. | Chamber overall dimensions, not less than | Width: 1400mm Length: 3000mm Height: 3000 mm |
| - 10. | Safety requirements | - The chamber (cabin) must comply with the EU standards for fire and electrical safety. |
| - 11. | Documentation | - The set of documentation should include instructions for the operation and maintenance of equipment, a repair manual with a list of names and coding of quickly worn replacement parts and assemblies, electrical diagram. - Language of technical documentation – Latvian and Russian. |
| - 12 | The warranty | - The warranty period is not less than 12 months. |